Enhet: 												Dato:

Deltakere ved kartleggingen (m/ funksjon):

Kort beskrivelse av hovedaktivitet/hovedprosess:

| ID nr. | Aktivitet/prosess | Ansvarlig | Lov, forskrift o.l. | Eksisterende dokumentasjon | Eksisterende sikringstiltak | Kommentar |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Acrylamid gel, making of |  |  | Procedure must be written before acrylamid is used | Nitril gloves, protective eyewear, ventilation hood | Procedure not currently performed |
| 2 | Agarosegel, making, running, Geldoc |  |  | Manual | Nitrile gloves, protective eyewear (with UV filter), face shield when needed. Gelred used for staining or alternatively blue and | Blue- and white screens stops UV-irradiation when mounted properly |
| 3 | Alginate isolation, purification and non-radioactive alginate assay |  |  | Protocols | Gloves, protective eyewear when using NaOH, Fume hood when using ether | Risk evaluation according to which chemicals are used |
| 4 | Antibiotic-stock solution, make and use of |  |  | Labrules, Relevant MSDSs | Gloves, handle powder only inside fume hood |  |
| 5 | Autoclave |  |  | Manual Driftsinstruks | Thermoresistant gloves Eyeprotection lab, Instructions posted for not opening autoclave too early and for not overfilling bottles or closing their lids completely. |  |
| 6 | Bacteria class 1 and recombinant bacteria |  |  | Approval: ”Godkjenning av laboratorier og anlegg for innesluttet bruk av genmodifiserte mikroorganismer” (08.12.10 Helsedirektoratet) | Autoclave accessible. Inactivation of genmanipulated bacteria in contaminated material and waste. Labcoat mandatory. Lab bench surfaces resistant to water, acid, alkali, solvents, disinfective agants, decontaminating agents and easy to clean. Transport between labs only in closed containers. Good microbiological practice. |  |
| 7 | Bacteria class 2 |  |  | Approval letters from arbeidstilsynet (11.06.10 and 27.04.07). | According to risk assesment: Risk assessments: “Novel anti-infective agents from marine bacteria (19.04.07)” and “Sintef project nr: 80117000” (14.011.08) and “Risikovurderinger og runtiner, arbeid med mikroorganismer i smitterisikogruppe 2” (28.05.09). | Room 309 and 429. Risk evaluation and procedures must be written for each experiment/new organism |
| 8 | Recombinant bacteria class 2 |  |  | Approval ”Godkjenning av laboratorier og anlegg for innesluttet bruk av genmodifiserte mikroorganismer” (08.12.10 Helsedirektoratet) | Autoclav in building. Inactivation of genmanipulated bacteria in contaminated material and waste. Labcoat and gloves mandatory. Lab bench surfaces resistant to water, acid, alkali, solvents, disinfective agants, decontaminating agents and easy to clean. Transport between labs only in closed containers. Restricted access to lab. Good microbiological practice. Biological hazard sign on door. Measures to minimize aerosols. Control of vectors (closed windows, no food I lab, general awareness of possible vectors), | Risk evaluation and procedures must be written for each experiment/new organism |
| 9 | Bioanalysator |  |  |  |  | No risk |
| 10 | Use of open flames – (e.g. sterilization with bunsen burners) |  |  | Labregler | Bunsen burner must not be left burning |  |
| 11 | Cellcracker (Mini bead beater) |  |  | Manual |  | No risk |
| 12 | General lab work |  |  | Laboratory handbook NTNU, MSDSs, EcoOnline system, signature for training/introduction to lab rules | Safety rules according to risk assessment |  |
| 13 | Development of film (Kodak Bio-Max film etc.) |  |  | Instructions on the bottles with concentrated chemicals (Developer and Fixer) | Gloves, glasses and labcoat. Work with precaution: no inhalation of fumes from Developer and Fixer. |  |
| 14 | DNA/RNA isolation and purification |  |  | Protocols | Use gloves and eyeprotection during steps including NaOH. Use fume hoods for procedures containing chloroform, phenol or if it is indicated in the kit manual |  |
| 15 | DNA/RNA kit for enzymatic modifications or clean up from reactions |  |  | Manuals | Use gloves or eyeprotection if the kit contains irritating fluids or proteases |  |
| 16 | Electroporator, use of |  |  | Manual |  | No risk |
| 17 | Environmental samples for cultivation and DNA isolation, preparation of |  |  | Risk assesment (projects) | According to risk assessment | Risk evaluation by specific project |
| 18 | Enzyme-assays (i.e. beta-lactamase-assay, phosphoglucomutase-assay, epimerase assay...) |  |  | Relevant protocols | According to MSDSs of relevant chemicals |  |
| 19 | Ethanol/dry ice bath |  |  | Labregler | Use gloves and eyeprotection | Potential damage to eyes and frostbite |
| 20 | FPLC, running of |  |  | Instructions for instrument. Precaution at instrument | According to MSDSs of relevant chemicals |  |
| 21 | Luciferace measurements, Promega Luciferase Assay kit |  |  | Manuals | According to MSDSs of relevant chemicals | No risk |
| 22 | Luminometer and fluorometer, use of |  |  | Manual | According to MSDSs of relevant chemicals | No risk |
| 23 | Mammalian cells, class 2 |  |  | Risk evaluation and procedures must be written for each experiment/new organism |  | Risk evaluation by specific project |
| 24 | Preparation of medias for growing bacteria |  |  | Recipe | According to MSDSs of relevant chemicals | No risk |
| 25 | Mutagenesis using a mutagenic agens (eg Etylmethanesulfonate, methylethanesulfonate, nitrosoguanidine, UV) |  |  | Experimental procedure Risk assessment and approval from project leader | Gloves, fume hoods, requires written procedure and risk assessment for the experimental prodcedure (type-specific) including detoxifying waste and how to warn and protect others working in the laboratory. Description and risk assessments must be written and approved by the responsible project leader (signature). Approval is mandatory not only for each experimental procedure, but also for each new person to perform that procedure |  |
| 26 | Nano-drop measurements of DNA/RNA |  |  | Manual |  | No risk |
| 28 | Northern and Southern blotting |  |  | Manual | According to MSDSs of relevant chemicals. Gloves and labcoat, work in a fume hood if MOPS is used. Usual precautions when working with UV light and DNA staining. | No risk |
| 29 | PCR |  |  | PCR-manual | Fume hood when DMSO is added |  |
| 30 | pH-adjustments |  |  | Manual | Nitril gloves, eyewear, shoes/shoe bags with protection agains acid/base spill, sufficient ventilation (hood/”cap”) |  |
| 31 | Protein isolation and purification |  |  |  | According to MSDSs of relevant chemicals | No risk (confer bacteria, sonication and chemicals used when applicable) |
| 32 | Proteingel-gels. Running of |  |  | CBS – manual http://cbsscientific.com/pdf/DCXInstructionsNov2010.pdf Sambrook and Russell, Molecular Cloning: A laboratory manual. section A.8.42 | Nitril gloves, eyewear, staining and destaining in fume hoods Protection against voltage (lid etc.) to avoid electical shock. |  |
| 33 | Pulse field electrophoresis |  |  | Manual Driftsinstruks | According to relevant MSDSs. Protection against voltage (lid, gloves etc.) to avoid electical shock. |  |
| 34 | Radioactivity work including use of scintillation counter |  |  | S:\Bio-alle\Radioaktivitet. Risk evaluation and procedures must be written for each experimental procedure Driftsinstruks scintillation counter | Gloves, eyeprotection, lab coat, hood. Testing of work area for contamination after work (filter). | The department has rules for being accepted as user of radioactive compounds and for having an experimental procedure accepted The scientillation liquid used may contain dangerous solvents |
| 35 | Sentrifugation (Sorvall + table) |  |  | Manual Driftsinstruks | Accurate balancing, accurate attachment of rotor, not exceed maximal G-forces fore each type of tube |  |
| 36 | Sonication |  |  | Manual Driftsinstruks | Ear protection. Pregnant women are not allowed to stay in the room during sonication. Sonication cabinet with sound absorbing material. Shut doors of the room where sonication is taking place |  |
| 37 | Supercompetent or electrocompetent cells, making of |  |  |  | Eye protection, gloves and protective shoes/shoe bags when handling liquid nitrogen or ethanol/dry ice bath. Shoes must be easy to take off in case of spill into (never use rubber boots) |  |
| 38 | Transformering (transformation) |  |  |  |  | No risk |
| 39 | Ultrasound waterbath, use of |  |  | Driftsinstruks | Ear protection. Pregnant women are not allowed to stay in the room during sonication |  |
| 40 | UV/Vis Spectrophotometer |  |  |  |  | No risk |
| 41 | Ventilation hoods, use of |  |  | Driftsinstruks | Opening minimized when not in use, correct settings when in used for protection |  |
| 42 | Washing machine, use of |  |  | Manual |  | No risk |
| 43 | Waterbaths and shaking waterbaths, (heat and water quality) |  |  | Manual | Timer on water baths |  |
| 44 | Western blotting |  |  |  | Chemicals |  |

Enhet: 													Dato: Dec 02 2010

Linjeleder:

Deltakere ved risikovurderingen (m/ funksjon): Helga Ertesvåg,

|  | Aktivitet fra kartleggings- skjemaet | Mulig uønsket hendelse/ belastning | Vurdering av sannsyn- lighet | Vurdering av konsekvens: |  |  |  | Risiko- verdi | Kommentarer/status Forslag til tiltak |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID nr |  |  | (1-5) | Menneske (A-E) | Ytre miljø (A-E) | Øk/ materiell (A-E) | Om- dømme (A-E) |  |  |
| 2 | Agarose gel electophoresis | Long term health risk from GelRed (unknown effect) | 1 | Unknown | A | A | B | Unknown | Gelred is said not to penetrate cell membranes, and thus should not act as mutagen even if it is DNA-binding. Gloves also minimize the risk for exposure. |
| 2 |  | UV-light: skin or eye damage | 2 | B | A | A | B | 2B | UV damages on unprotected skins/eyes if instructions not followed |
| 5 | Autoclave | Rapid pressure fall due to opening the autoclave to soon my cause hot liquid burns on eye or skin | 2 | C | A | A | B | 2C | Will not happen if instructions are followed |
| 6 | Bacteria class 1 and recombinant bacteria | Release of GMO to environment Bacterial infections | 2 | A | A | A | B | 2A |  |
| 7 | Bacteria class 2 | Bacterial infections. Release of infective, pathogen bacteria to environment. | 2 | C | B | A | C | 2C | Low risk because of strict regulations and good lab practise. |
| 8 | Recombinant bacteria class 2 | Bacterial infections. Release of infective, pathogen gene manipulated bacteria to environment. | 2 | C | B | A | C | 2C | Low risk because of strict regulations and good lab practice. |
| 10 | Bunsen burners | Skin burns | 3 | B | A | A | A | 3B | New rules how to handle burners+ ethanol baths already installed. |
| 10 | Bunsen burners | Initiation of fire. | 3 | C | A | B | A | 3C | New rules how to handle burners + ethanol baths already installed. |
| 12 | General lab work | Deviation from established safety rules and routines | -- | -- | -- | -- | -- | -- | No injuries requiring more than simple first aid in these laboratories for the past 5 years, the present rutines seem sufficient |
| 13 | Development of film (Kodak Bio-Max film etc.) | Exposure to Developer/Fixative chemicals. Inhalation must be avoided | 2 | B | A | A | B | 2B | –All usual lab work routines have to be followed (gloves, glasses, lab coat) |
| 14 | DNA/RNA isolation and purification | Exposure for dangerous for health chemicals: Phenol-chlorophorm mix | 2 | C | A | A | B | 2C | Phenol-chloroform mix requires the work in a ventilation hood only. All waste should be placed in a special box for hazardous materials. |
| 17 | Environmental samples for cultivation and DNA isolation, preparation of | Bacterial infections, production of toxic/infectous compunds from cloned DNA. Usage of hazardous compounds (e.g. chloroform) for DNA isolation. | 1 | unknown | A | A | C | 1 - unknown | In general mild injuries, that can be avoided by present rutines and senseable carefulness during lab-work. |
| 19 | Dry ice/etanol | Eye injury. Frostbite | 1 | B | A | A | B | 1B | Eye-protection and Gloves for handling the Dry ice |
| 20 | FPLC, running of | Leakage of buffer | 2 | See MSDS | A | A | Depends on injury | 1A |  |
| 22 | Luminometer and fluorometer, use of | Possible exposure health injurious chemicals. | 2 | A | A | A | A | 2A | See MSDS for relevant chemicals |
| 23 | Mammalian cells, class 2 | Exposure to biological agents, infections. | 2 | D | A | A | D | 2D | Applying aseptic procedures. Good microbiological practices. |
| 25 | Mutagenesis using a mutagenic agens (eg Etylmethanesulfonate, methylethanesulfonate, nitrosoguanidine, UV) | Exposure to mutagens | 1 | E | A | A | E | 1E | Probably too small amounts to ever damage the environment. Because of the known risk of cancer, these experiments are always conducted with extreme caution, and using gloves and fume hoods. |
| 28 | Northern/Southern | Exposure to injourous chemicals. Northern blot may involve formaldehyde and acrylamide which are toxic, thus fume hood and protective clothing must be used. Due to the use of toxic chemicals special disposal mothods are required. | 1 | C | A | A | A | 1C |  |
| 29 | PCR | Exposure to DMSO when used | 1 | A | A | A | A | 1A | PCR-machine should be in ventilation hood when DMSO is added, use lab coat and gloves |
| 30 | pH-adjustments | Exposure to corrosive chemicals: Skin and eye damage | 2 | B | A | A | B | 2B | Eye-protection and lab coat |
| 32 | Protein geleletroforese | Electrical shock | 1 | E | A | A | C | 1E |  |
| 33 | Pulse field electrophoresis | Electrical shock and depending on method used for DNA preparation:) | 1 | E | A | A | A | 1E | Equipment for electrophoresis will only work when lid is closed, thus minimizing the possibility of suffering an electric shock. |
|  |  | Exposure to hazardous chemical: phenylmethanesulphonyl fluoride (toxic, corrosive) | 1 | C | A | A | B | 1C | Use of gloves/fume hoods for handling of hazardous chemicals will minimize risk. |
| 35 | Sentrifugation (Sorvall + table) | Damage caused by loose rotor | 1 | C | A | B | C | 1C |  |
| 34 | Radioactivity work including use of scintillation counter | Health injury as result of radiation (external, internal as result of uptake). Radioactive contamination of working environment. | 1 | E | A | A | E | 1E |  |
| 36 | Sonication | Damage to hearing (including fetus) | 1 | C | A | A | B | 1C |  |
|  |  | Creation of injourous aerosols (infectious microorganisms, toxic agents) |  |  |  |  |  |  |  |
| 37 | Supercompetent or electrocompetent cells, making of | Exposure of skin and eyes to extremely cold liquid (nitrogen or ethanol) | 2 | C | A | A | B | 2C | Conducted with extreme caution, and using eye-protection and gloves. |
| 39 | Ultrasound waterbath, use of | Damage to hearing (including fetus) | 1 | C | A | A | B | 1C |  |
| 41 | Ventilation hoods, use of | Exposure to hazardous chemicals due to unsufficient airflow (effects on local hood or other hoods) | 1 | C | A | A | B | 1C | Can be prevented by maintaining sufficient airflow |
| 43 | Waterbaths and shaking waterbaths, (heat and water quality) | Electrical hazards, Burns | 1 | B | B | A | A | 1B | Regular service by trained personnel. Heat-proof gloves must be used during handling of hot items. Hot surfaces must be signed. |
| 44 | Western blotting | Exposure to hazardous chemicals | 1 | See MSDS | A | A | Depends on injury | 1A |  |

|  | Sannsynlighet | Konsekvens | Risikoverdi (beregnes hver for seg): |
| --- | --- | --- | --- |
|  | 1. Svært liten 2. Liten 3. Middels 4. Stor 5. Svært stor | A. Svært liten B. Liten C. Moderat D. Alvorlig E. Svært alvorlig | Menneske = Sannsynlighet x Konsekvens Menneske Ytre miljø = Sannsynlighet x Konsekvens Ytre miljø Økonomi/materiell = Sannsynlighet x Konsekvens Øk/matriell Omdømme = Sannsynlighet x Konsekvens Omdømme |

Sannsynlighet vurderes etter følgende kriterier:

| Svært liten 1 | Liten 2 | Middels 3 | Stor 4 | Svært stor 5 |
| --- | --- | --- | --- | --- |
| 1 gang pr 50 år eller sjeldnere | 1 gang pr 10 år eller sjeldnere | 1 gang pr år eller sjeldnere | 1 gang pr måned eller oftere | Skjer ukentlig |

Konsekvens vurderes etter følgende kriterier:

| Gradering | Menneske | Ytre miljø Vann, jord og luft | Øk/materiell | Omdømme |
| --- | --- | --- | --- | --- |
| E Svært Alvorlig | Død | Svært langvarig og ikke reversibel skade | Drifts- eller aktivitetsstans >1 år. | Troverdighet og respekt betydelig og varig svekket |
| D Alvorlig | Alvorlig personskade. Mulig uførhet. | Langvarig skade. Lang restitusjonstid | Driftsstans > ½ år Aktivitetsstans i opp til 1 år | Troverdighet og respekt betydelig svekket |
| C Moderat | Alvorlig personskade. | Mindre skade og lang restitusjonstid | Drifts- eller aktivitetsstans < 1 mnd | Troverdighet og respekt svekket |
| B Liten | Skade som krever medisinsk behandling | Mindre skade og kort restitusjonstid | Drifts- eller aktivitetsstans < 1uke | Negativ påvirkning på troverdighet og respekt |
| A Svært liten | Skade som krever førstehjelp | Ubetydelig skade og kort restitusjonstid | Drifts- eller aktivitetsstans < 1dag | Liten påvirkning på troverdighet og respekt |

Risikoverdi = Sannsynlighet x Konsekvens

Beregn risikoverdi for Menneske. Enheten vurderer selv om de i tillegg vil beregne risikoverdi for Ytre miljø, Økonomi/materiell og Omdømme. I så fall beregnes disse hver for seg.

Til kolonnen ”Kommentarer/status, forslag til forebyggende og korrigerende tiltak”:

Tiltak kan påvirke både sannsynlighet og konsekvens. Prioriter tiltak som kan forhindre at hendelsen inntreffer, dvs. sannsynlighetsreduserende tiltak foran skjerpet beredskap, dvs. konsekvensreduserende tiltak.