Unit: 	IBT, NTNU										Date: 	20.10.11

Participants in the identification process (including their function):Martin Gimmestad, HSE-coordinator

Short description of the main activity/main process: TBT4145 lab course

| ID nr. | Activity/process | Responsible person | Laws, regulations etc. | Existing documentation | Existing safety measures | #### Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Agarosegel, making, running, Geldoc |  |  | Manual | Nitrile gloves, protective eyewear (with UV filter), face shield when needed. Gelred used for staining |  |
| 2 | Antibiotic-stock solution, make and use of |  |  | Labroules, Relevant MSDSs | Gloves, handle powder only inside fume hood |  |
| 3 | Bacteria class 1 and recombinant bacteria |  |  | Approval: ”Godkjenning av laboratorier og anlegg for innesluttet bruk av genmodifiserte mikroorganismer” (08.12.10 Helsedirektoratet) | Autoclave accessible. Inactivation of genmanipulated bacteria in contaminated material and waste. Labcoat mandatory. Lab bench surfaces resistant to water, acid, alkali, solvents, disinfective agants, decontaminating agents and easy to clean. Transport between labs only in closed containers. Good microbiological practice. |  |
| 4 | Use of open flames – (e.g. sterilization with bunsen burners) |  |  |  | Bunsen burner must not be left burning |  |
| 5 | Plasmid islation and purification |  |  | Protocols | Use gloves, labcoat and eyeprotection. Avoid contact with skin, eyes. |  |
| 10 | Sentrifugation (Sorvall + table) |  |  | Manual Driftsinstruks | Accurate balancing, accurate attachment of rotor, not exceed maximal G-forces fore each type of tube |  |
| 11 | TSS transformasjon |  |  | Manual | Wear gloves, avoid exposure to skin, eyes | TSS contains DMSO |

Unit: 		IBT, NTNU											Date: 	20.10.11

Line manager:Kjetil Rasmussen

Participants in the risk assessment (including their function):Martin Gimmestad, HSE-coordinator

| ID nr | Activity from the identification process form | Potential undesirable incident/strain | Likelihood: | Consequence: |  |  | Risk value | Comments/status Suggested measures |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Likelihood (1-5) | Human (A-E) | Environment (A-E) | Economy/ material (A-E) |  |  |
| 1 | Agarose gel electophoresis | Long term health risk from GelRed (unknown effect) | 1 | Unknown | A | A | Unknown | Gelred is said not to penetrate cell membranes, and thus should not act as mutagen even if it is DNA-binding. Gloves also minimize the risk for exposure. |
|  |  | UV-light: skin or eye damage | 2 | B | A | A | 2B | UV damages on unprotected skins/eyes if instructions not followed |
| 2 | Antibiotic-stock solution, make and use of | Exposure to biologically active compounds | 2 | B | A | A | 2B |  |
| 3 | Bacteria class 1 and recombinant bacteria | Release of GMO to environment Bacterial infections | 2 | A | A | A | 2A |  |
| 4 | Use of open flames – (e.g. sterilization with bunsen burners) | Skin burns | 3 | B | A | A | 3B |  |
|  |  | Initiation of fire. | 3 | C | A | C | 3C |  |
| 5 | DNA isolation and purification | Exposure to corrosive and irritating chemicals in lysis- and neutralization solutions, alkaline protease solution | 2 | C | A | A | 2C |  |
| 6 | Sentrifugation (Sorvall + table) | Imbalance, loosening of rotor | 1 | B | A | A | 1B |  |
| 7 | TSS transformasjon | Exposure to DMSO | 3 | Unknown | A | A | Unknown | DMSO makes membranes more permeable. Avoid contact. Be careful. |

| Likelihood, e.g.: | Consequence, e.g.: | Risk value (each one to be estimated separately): |
| --- | --- | --- |
| - Minimal - Low - Medium - High - Very high | A. Safe B. Relatively safe C. Dangerous D. Critical E. Very critical | Human = Likelihood x Human Consequence Environmental = Likelihood x Environmental consequence Financial/material = Likelihood x Consequence for Economy/materiel |

Potential undesirable incident/strain

Identify possible incidents and conditions that may lead to situations that pose a hazard to people, the environment and any materiel/equipment involved.

Criteria for the assessment of likelihood and consequence in relation to fieldwork

Each activity is assessed according to a worst-case scenario. Likelihood and consequence are to be assessed separately for each potential undesirable incident. Before starting on the quantification, the participants should agree what they understand by the assessment criteria:

Likelihood

| Minimal 1 | Low 2 | Medium 3 | High 4 | Very high 5 |
| --- | --- | --- | --- | --- |
| Once every 50 years or less | Once every 10 years or less | Once a year or less | Once a month or less | Once a week |

Consequence

| Grading | Human | Environment | Financial/material |
| --- | --- | --- | --- |
| E Very critical | May produce fatality/ies | Very prolonged, non-reversible damage | Shutdown of work >1 year. |
| D Critical | Permanent injury, may produce serious serious health damage/sickness | Prolonged damage. Long recovery time. | Shutdown of work 0.5-1 year. |
| C Dangerous | Serious personal injury | Minor damage. Long recovery time | Shutdown of work < 1 month |
| B Relatively safe | Injury that requires medical treatment | Minor damage. Short recovery time | Shutdown of work < 1week |
| A Safe | Injury that requires first aid | Insignificant damage. Short recovery time | Shutdown of work < 1day |

The unit makes its own decision as to whether opting to fill in or not consequences for economy/materiel, for example if the unit is going to use particularly valuable equipment. It is up to the individual unit to choose the assessment criteria for this column.

Risk = Likelihood x Consequence

Please calculate the risk value for “Human”, “Environment” and, if chosen, “Economy/materiel”, separately.

About the column ”Comments/status, suggested preventative and corrective measures”:

Measures can impact on both likelihood and consequences. Prioritise measures that can prevent the incident from occurring; in other words, likelihood-reducing measures are to be prioritised above greater emergency preparedness, i.e. consequence-reducing measures.

Risk assessment matrix – NTNU

Green = acceptable risk ; yellow = moderate risk ; red = unacceptable risk