Enhet: 												Dato:

Deltakere ved kartleggingen (m/ funksjon):

Kort beskrivelse av hovedaktivitet/hovedprosess:

| ID nr. | Aktivitet/prosess | Ansvarlig | Lov, forskrift o.l. | Eksisterende dokumentasjon | Eksisterende sikringstiltak | Kommentar |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | General lab work | ES, OD |  | Laboratory handbook NTNU, MSDSs, EcoOnline system, SABA safety rules, NMR laboratory safety rules, signature for training/introduction to SABA/NMR rules | Safety rules according to risk assessment |  |
| 2 | Supercompetent or electrocompetent cells, making of | ES, OD |  |  | Eye protection, gloves and protective shoes/shoe bags when handling liquid nitrogen or ethanol/dry ice bath. Shoes must be easy to take off in case of spill into (never use rubber boots) |  |
| 3 | Stock solution, make and use of | ES, OD |  | Labroules, Relevant MSDSs, Recipe | Gloves, handle powder only inside fume hood for volatile or hazard substances | respiratory tract irritation |
| 4 | Storage of biomaterial | ES, OD |  | SABA laboratoty rules, manual | Gloves and protective shoes/shoe bags when handling liquid nitrogen or ethanol/dry ice bath. Never go into the basement without warning other person. |  |
| 5 | Preparing of steril media and solution | ES, OD |  | SABA laboratoty rules, manual, Recipe | Weigh the volatile substances under ventilation hood Never fill a full bottle with solution Never close tightly the bottle | respiratory tract irritation Solution pouring Explosion |
| 6 | Transformering (transformation) | ES, OD |  |  | Check all cords of waterbath before use, since corrosive chemicals or solvents may erode the insulation. Check if deionized water cover the heater of waterbath. | Electrical shock and fire |
| 7 | Working with glassware | ES, OD |  |  | Do not touch broken glassware with your hands Use for sterilisation and vacuum only round-bottomed or thick-walled (e.g., Pyrex) glassware | glass vessels may collapse violently |
| 8 |  |  |  |  |  |  |
| 8 | Autoclave | ES, OD |  | Manual Driftsinstruks | Thermoresistant gloves Eyeprotection lab, Instructions posted for fill autoclave with deionised water (were it’s required) not opening autoclave too early and for not overfilling bottles or closing their lids completely. |  |
| 9 | Bacteria class 1 and recombinant bacteria | ES, OD |  | Approval: ”Godkjenning av laboratorier og anlegg for innesluttet bruk av genmodifiserte mikroorganismer” (08.12.10 Helsedirektoratet) | Autoclave accessible. Inactivation of genmanipulated bacteria in contaminated material and waste. Labcoat mandatory. Lab bench surfaces resistant to water, acid, alkali, solvents, disinfective agants, decontaminating agents and easy to clean. Transport between labs only in closed containers. Good microbiological practice. |  |
| 10 | Use of open flames – (e.g. sterilization with bunsen burners, making glass pipets) | ES, OD |  | Labregler | Bunsen burner must not be left burning, be creful with hot glass |  |
| 11 | Cellcracker (Mini bead beater) | ES, OD |  | Manual |  | No risk |
| 12 | DNA/RNA isolation and purification | ES, OD |  | Protocols | Use gloves and eyeprotection during steps including NaOH. Use fume hoods for procedures containing chloroform, phenol or if it is indicated in the kit manual |  |
| 13 | Cooling Baths and Dry Ice: Ethanol(Aceton)/dry ice bath | ES, OD |  | Labregler | Use gloves and eyeprotection Do not lower your head into a dry ice chest | Potential damage to eyes and frostbite suffocation from carbon dioxide buildup |
| 14 | Cold room use | ES, OD |  |  | Do not stored or leave ON electrical apparatouses | Corrosion |
| 15 | Vacuum | ES, OD |  | Manual | safety glasses or chemical goggles, face shields Use vacuum trap | Explosion vapors and related odors can be emitted back into the laboratory or system exhaust |
| 16 | Heating Devices | ES, OD |  |  | Do not leave ON without controll | Burn |
| 17 | FPLC, running of | ES, OD |  | Instructions for instrument. Precaution at instrument | According to MSDSs of relevant chemicals Use appropriate load speed for each column Check the pressure in the system and the connection of elements | Leaking of liquids |
| 18 | Nano-drop measurements of DNA/RNA | ES, OD |  | Manual |  | No risk |
| 19 | pH-adjustments | ES, OD |  | Manual | Nitril gloves, eyewear, shoes/shoe bags with protection agains acid/base spill, sufficient ventilation (hood/”cap”) |  |
| 20 | Protein isolation and purification | ES, OD |  |  | According to MSDSs of relevant chemicals | No risk (confer bacteria, sonication and chemicals used when applicable) |
| 21 | Proteingel-gels. Running of | ES, OD |  | Manual Invitrogen Sambrook and Russell, Molecular Cloning: A laboratory manual. section A.8.42 | Nitril gloves, eyewear, staining and destaining in fume hoods Protection against voltage (lid etc.) to avoid electical shock. |  |
| 22 | Sentrifugation (Sorvall + table) | ES, OD |  | Manual Driftsinstruks | Accurate balancing, accurate attachment of rotor, not exceed maximal G-forces fore each type of tube Use only suitable type of tubes |  |
| 23 | Sonication | ES, OD |  | Manual Driftsinstruks | Ear protection. Pregnant women are not allowed to stay in the room during sonication. Sonication cabinet with sound absorbing material. Shut doors of the room where sonication is taking place. |  |
| 24 | Ultrasound waterbath, use of | ES, OD |  | Driftsinstruks | Ear protection. Pregnant women are not allowed to stay in the room during sonication |  |
| 25 | UV/Vis Spectrophotometer | ES, OD |  |  |  | No risk |
| 26 | Ventilation hoods, use of | ES, OD |  | Driftsinstruks | Opening minimized when not in use, correct settings when in used for protection |  |
| 27 | Microwave Ovens | ES, OD |  |  | Do not use metal containers and metal-containing objects (e.g., stir bars) in the microwave Do not heat sealed containers in the microwave oven | Arcing Explosion |
| 28 | Gas | ES, OD |  |  | Avoid dropping, dragging or sliding cylinders Use only wrenches or tools provided by the cylinder supplier to remove a cylinder cap or to open a valve. Before using cylinders, read all label information and material safety data sheets (MSDSs) associated with the gas being used. |  |
| 29 | Washing machine, use of | ES, OD |  | Manual |  | No risk |
| 30 | NMR spectromentr | ES, OD |  | Manual of NMR centre | General rules for NMR equipment |  |
| 31 | Waterbaths and shaking waterbaths, (heat and water quality) | ES, OD |  | Manual | Timer on water baths |  |
| 32 | Western blotting | ES, OD |  |  | Chemicals |  |
| 33 | Corrosive Liquids | ES, OD |  |  | Use gloves and eyeprotection Always add acids or bases to water (and not the reverse) Liquid corrosives should be stored below eye level | flash steam explosion |

Enhet: 													Dato: 11 11 2011

Linjeleder:

Deltakere ved risikovurderingen (m/ funksjon): Elena Shumilina,

|  | Aktivitet fra kartleggings- skjemaet | Mulig uønsket hendelse/ belastning | Vurdering av sannsyn- lighet | Vurdering av konsekvens: |  |  |  | Risiko- verdi | Kommentarer/status Forslag til tiltak |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ID nr |  |  | (1-5) | Menneske (A-E) | Ytre miljø (A-E) | Øk/ materiell (A-E) | Om- dømme (A-E) |  |  |
| 1 | Autoclave | Rapid pressure fall due to opening the autoclave to soon my cause hot liquid burns on eye or skin | 2 | C | A | A | B | 2C | Will not happen if instructions are followed |
| 2 | Bacteria class 1 and recombinant bacteria | Release of GMO to environment Bacterial infections | 2 | A | A | A | B | 2A |  |
| 3 | Bunsen burners | Skin burns | 3 | B | A | A | A | 3B | New rules how to handle burners+ ethanol baths already installed. |
| 4 | Bunsen burners | Initiation of fire. | 3 | C | A | B | A | 3C | New rules how to handle burners + ethanol baths already installed. |
| 5 | General lab work | Deviation from established safety rules and routines | -- | -- | -- | -- | -- | -- | No injuries requiring more than simple first aid in these laboratories for the past 5 years, the present rutines seem sufficient |
| 6 | DNA/RNA isolation and purification | Exposure for dangerous for health chemicals: Phenol-chlorophorm mix | 2 | C | A | A | B | 2C | Phenol-chloroform mix requires the work in a ventilation hood only. All waste should be placed in a special box for hazardous materials. |
| 7 | Dry ice/etanol(acetone) | Eye injury. Frostbite | 1 | B | A | A | B | 1B | Eye-protection and Gloves for handling the Dry ice |
| 8 | FPLC, running of | Leakage of buffer | 2 | See MSDS | A | A | Depends on injury | 1A |  |
| 9 | pH-adjustments | Exposure to corrosive chemicals: Skin and eye damage | 2 | B | A | A | B | 2B | Eye-protection and lab coat |
| 10 | Protein geleletroforese | Electrical shock | 1 | E | A | A | C | 1E |  |
| 11 |  | Exposure to hazardous chemical: phenylmethanesulphonyl fluoride (toxic, corrosive) | 1 | C | A | A | B | 1C | Use of gloves/fume hoods for handling of hazardous chemicals will minimize risk. |
| 12 | Sentrifugation (Sorvall + table) | Damage caused by loose rotor | 1 | C | A | B | C | 1C |  |
| 13 | Sonication | Damage to hearing (including fetus) | 1 | C | A | A | B | 1C |  |
| 14 |  | Creation of injourous aerosols (infectious microorganisms, toxic agents) |  |  |  |  |  |  |  |
| 15 | Supercompetent or electrocompetent cells, making of | Exposure of skin and eyes to extremely cold liquid (nitrogen or ethanol) | 2 | C | A | A | B | 2C | Conducted with extreme caution, and using eye-protection and gloves. |
| 16 | Ultrasound waterbath, use of | Damage to hearing (including fetus) | 1 | C | A | A | B | 1C |  |
| 17 | Ventilation hoods, use of | Exposure to hazardous chemicals due to unsufficient airflow (effects on local hood or other hoods) | 1 | C | A | A | B | 1C | Can be prevented by maintaining sufficient airflow |
| 18 | Waterbaths and shaking waterbaths, (heat and water quality) | Electrical hazards, Burns | 1 | B | B | A | A | 1B | Regular service by trained personnel. Heat-proof gloves must be used during handling of hot items. Hot surfaces must be signed. |

|  | Sannsynlighet | Konsekvens | Risikoverdi (beregnes hver for seg): |
| --- | --- | --- | --- |
|  | 1. Svært liten 2. Liten 3. Middels 4. Stor 5. Svært stor | A. Svært liten B. Liten C. Moderat D. Alvorlig E. Svært alvorlig | Menneske = Sannsynlighet x Konsekvens Menneske Ytre miljø = Sannsynlighet x Konsekvens Ytre miljø Økonomi/materiell = Sannsynlighet x Konsekvens Øk/matriell Omdømme = Sannsynlighet x Konsekvens Omdømme |

Sannsynlighet vurderes etter følgende kriterier:

| Svært liten 1 | Liten 2 | Middels 3 | Stor 4 | Svært stor 5 |
| --- | --- | --- | --- | --- |
| 1 gang pr 50 år eller sjeldnere | 1 gang pr 10 år eller sjeldnere | 1 gang pr år eller sjeldnere | 1 gang pr måned eller oftere | Skjer ukentlig |

Konsekvens vurderes etter følgende kriterier:

| Gradering | Menneske | Ytre miljø Vann, jord og luft | Øk/materiell | Omdømme |
| --- | --- | --- | --- | --- |
| E Svært Alvorlig | Død | Svært langvarig og ikke reversibel skade | Drifts- eller aktivitetsstans >1 år. | Troverdighet og respekt betydelig og varig svekket |
| D Alvorlig | Alvorlig personskade. Mulig uførhet. | Langvarig skade. Lang restitusjonstid | Driftsstans > ½ år Aktivitetsstans i opp til 1 år | Troverdighet og respekt betydelig svekket |
| C Moderat | Alvorlig personskade. | Mindre skade og lang restitusjonstid | Drifts- eller aktivitetsstans < 1 mnd | Troverdighet og respekt svekket |
| B Liten | Skade som krever medisinsk behandling | Mindre skade og kort restitusjonstid | Drifts- eller aktivitetsstans < 1uke | Negativ påvirkning på troverdighet og respekt |
| A Svært liten | Skade som krever førstehjelp | Ubetydelig skade og kort restitusjonstid | Drifts- eller aktivitetsstans < 1dag | Liten påvirkning på troverdighet og respekt |

Risikoverdi = Sannsynlighet x Konsekvens

Beregn risikoverdi for Menneske. Enheten vurderer selv om de i tillegg vil beregne risikoverdi for Ytre miljø, Økonomi/materiell og Omdømme. I så fall beregnes disse hver for seg.

Til kolonnen ”Kommentarer/status, forslag til forebyggende og korrigerende tiltak”:

Tiltak kan påvirke både sannsynlighet og konsekvens. Prioriter tiltak som kan forhindre at hendelsen inntreffer, dvs. sannsynlighetsreduserende tiltak foran skjerpet beredskap, dvs. konsekvensreduserende tiltak.