- GUIDELINES FOR USE OF BRANSON SONIFIER

- Safety precautions:

Pregnant women should not be exposed to ultrasonic energy. Do not operate the sonifier if you or other persons present in the room is pregnant. Always ask for permission before starting the machine.

- Instructions for use:

- If you have not used the equipment before, contact your supervisor or Espen (phone 93217810) for instructions.

- Always log in.

- Be sure that your sample container can withstand the ultrasonic energy. Tubes of polypropylene (PP) are OK. Only tempered glass should be used.

- Make sure the tip is attached correctly. Instructions for tip assembly are on the wall. Contact supervisor or Espen if in doubt.

- Close the door of the cabinet and put on earmuffs. If anyone else is present, ask them to leave or make them put on earmuffs.

- Wash the tip with 70% ethanol prior to use.

- When you are finished, remove any spill and wash with 70% ethanol, wipe off with paper.

- Espen Fjærvik

espen.fjarvik@biotech.ntnu.no

Phone: 73598683 / 93217810

Room: D1-173

2010-12-22